# Arduino Code

/*

Spider Bot – Basic Navigation Code

Hardware: Arduino UNO (or compatible) + L298N + 2 DC motors + battery

Motion provided:

- forward(durationMs)

- backward(durationMs)

- turnLeft(durationMs)

- turnRight(durationMs)

- stopMotors(durationMs)

- stopMotors() [immediate stop, no delay]

Assumed connections (change pin numbers if your wiring is different):

Left motor:

L298N IN1 -> D8

L298N IN2 -> D9

L298N ENA -> D5 (PWM)

Right motor:

L298N IN3 -> D10

L298N IN4 -> D11

L298N ENB -> D6 (PWM)

Both motors share common GND with Arduino and battery.

*/

///////////////////////

// Pin Definitions  //

///////////////////////

// Left motor pins

const int IN1 = 8;    // L298N IN1

const int IN2 = 9;    // L298N IN2

const int ENA = 5;    // L298N ENA (PWM)

// Right motor pins

const int IN3 = 10;   // L298N IN3

const int IN4 = 11;   // L298N IN4

const int ENB = 6;    // L298N ENB (PWM)

// Motor speed (0–255). Adjust based on your motors and battery.

int motorSpeed = 180;

void setup() {

// Set all control pins as outputs

pinMode(IN1, OUTPUT);

pinMode(IN2, OUTPUT);

pinMode(ENA, OUTPUT);

pinMode(IN3, OUTPUT);

pinMode(IN4, OUTPUT);

pinMode(ENB, OUTPUT);

// Initially stop

stopMotors();

// Optional: Serial for debugging

Serial.begin(9600);

Serial.println("Spider Bot navigation started.");

}

void loop() {

// Example demonstration sequence:

Serial.println("Forward 2 seconds");

forward(2000);        // move forward for 2 s

stopMotors(500);      // stop for 0.5 s

Serial.println("Backward 1.5 seconds");

backward(1500);       // move backward for 1.5 s

stopMotors(500);

Serial.println("Turn left 1 second");

turnLeft(1000);       // turn left for 1 s

stopMotors(500);

Serial.println("Turn right 1 second");

turnRight(1000);      // turn right for 1 s

stopMotors(1500);

// Repeat

}

////////////////////////

// Motion Functions  //

////////////////////////

// Forward for a specified duration (ms)

void forward(unsigned long durationMs) {

// Left motor forward

digitalWrite(IN1, HIGH);

digitalWrite(IN2, LOW);

analogWrite(ENA, motorSpeed);

// Right motor forward

digitalWrite(IN3, HIGH);

digitalWrite(IN4, LOW);

analogWrite(ENB, motorSpeed);

delay(durationMs);

}

// Backward for a specified duration (ms)

void backward(unsigned long durationMs) {

// Left motor backward

digitalWrite(IN1, LOW);

digitalWrite(IN2, HIGH);

analogWrite(ENA, motorSpeed);

// Right motor backward

digitalWrite(IN3, LOW);

digitalWrite(IN4, HIGH);

analogWrite(ENB, motorSpeed);

delay(durationMs);

}

// Turn left (approx. on the spot) for a specified duration (ms)

void turnLeft(unsigned long durationMs) {

// Left motor backward

digitalWrite(IN1, LOW);

digitalWrite(IN2, HIGH);

analogWrite(ENA, motorSpeed);

// Right motor forward

digitalWrite(IN3, HIGH);

digitalWrite(IN4, LOW);

analogWrite(ENB, motorSpeed);

delay(durationMs);

}

// Turn right (approx. on the spot) for a specified duration (ms)

void turnRight(unsigned long durationMs) {

// Left motor forward

digitalWrite(IN1, HIGH);

digitalWrite(IN2, LOW);

analogWrite(ENA, motorSpeed);

// Right motor backward

digitalWrite(IN3, LOW);

digitalWrite(IN4, HIGH);

analogWrite(ENB, motorSpeed);

delay(durationMs);

}

// Stop both motors immediately (no delay)

void stopMotors() {

// Disable left motor

digitalWrite(IN1, LOW);

digitalWrite(IN2, LOW);

analogWrite(ENA, 0);

// Disable right motor

digitalWrite(IN3, LOW);

digitalWrite(IN4, LOW);

analogWrite(ENB, 0);

}

// Stop both motors and hold that state for durationMs

void stopMotors(unsigned long durationMs) {

stopMotors();

delay(durationMs);

}